ECE 3720

PWM (Pulse Width Modulation)

Section 2

Brandon Wider

Date Performed: March / 10 / 2020

Lab 8

ABSTRACT:

This lab discusses how to use the PIC32’s comparator in PWM mode to produce a signal to power a DC motor at different speeds.

INTRODUCTION:

The purpose of this lab is to program the microcontroller to power a DC motor with different voltage levels, causing the motor to operate at different speeds.  The PIC32 doesn’t have a PWM, so we use the comparator to act like a PWM.  The speed is controlled by changing the duty cycle of the PWM, which changes the average voltage seen by the motor.  A debouncer is used to cycle through different duty cycles values.

EXPERIMENTAL PROCEDURES:

- Wire circuit using the crude block diagram

- For the interrupt part of the code, cycle through to the next speed by writing the new duty cycle to the OSxRS register.

RESULTS and DISCUSSION:

When done correctly, motor will initially be off, and then each time the debouncer switch is pressed the duty cycle is cycled through 0%, 25%, 50%, 75%, and 100%.  The duty cycle determines the average voltage received by the DC motor, which determines the motors speed. Even though the PIC32 doesn’t have a PWM, the comparator can be used to act just like it. This is done by switching the signal on and off whenever the clock counter is equal to the duty cycle.

CONCLUSSION:

This lab shows how to implement a PWM in the PIC32 by using the comparator, and how to change the duty cycle of the PWM while the program’s running.

REFERENCES:

Clemson University’s ECE 372 Lab 8 Powerpoint.

FIGURES AND TABLES:

CODE:

#include <plib.h>

int count;
void __ISR(3)debuncer() {
    count += 25;
    if (count > 100)
        count = 0;
    OC4RS = count;
    IFS0bits.INT0IF = 0;
}

main() {
    PPSOutput(3, RPB6, OC4);
    INTEnableSystemMultiVectoredInt();
    INTCONbits.INT0EP = 1; //Enable INT0 to happen
    IPC0bits.INT0IP = 1; //Set priority control
    IEC0bits.INT0IE = 1; //Enable interrupt 0
    IFS0bits.INT0IF = 0; //Sets up interrupt flag status

    int count = 0;
    T2CON = 0;
    T2CONbits.TCS = 0;
    T2CONbits.TCKPS = 1;
    T2CONbits.TGATE = 0;
    TMR2 = 0;
    PR2 = 100;
    T2CONbits.ON = 1;
   
    TRISBbits.TRISB0 = 0; //LED 1
    TRISBbits.TRISB1 = 0; //LED 2
    TRISBbits.TRISB2 = 0; //LED 3
    TRISBbits.TRISB3 = 0; //LED 4
   
    TRISBbits.TRISB7 = 1;
    TRISBbits.TRISB6 = 0;
   
    OC4RS = 50;
    OC4CONbits.ON = 0;
    OC4CONbits.OC32 = 0;
    OC4CONbits.OCM = 7;
    OC4CONbits.OCTSEL = 0;
    OC4CONbits.ON = 1;
    while (1){
    }
}